Print the settings to compare (OKIDATA320):

1 Make sure paper is loaded in the printer.

2 Enter the Menu Mode (SHIFT + SEL).

3 Press PRINT (PARK key).

If the invoice does not start print on the correct line:
• Press the "PARK" button. This will unload the paper.
• Make sure that the paper is in the paper tractor properly.
• Press the "FF/LOAD" button which will load the paper.
Please make sure that we don't adjust the paper (very important!) after the paper is loaded with the "FF/LOAD" button.
• Try to print the Order/Invoice again.

If it still doesn't align loaded with "FF/LOAD":
• Hold "SHIFT" and "SEL" simultaneously until the menu light turns on.
• Once the menu light is on hold down the "Micro feed Down" (also labeled as LF) until the paper goes up or down this jumps the paper 6 lines max.
• Exit the menu by pressing the "SHIFT" and "SEL". [This will save permanent].
• Try to print the Order/Invoice again. Please make sure we don't adjust the paper after or before by the handle (nob) not even to unload the paper. If we did just hit "PARK" than "FF/LOAD"  described above.

If  the adjustment above moved the paper to the wrong direction do the same steps except use "Micro Feed Up" (also labeled as FF/LOAD) option. The main trick on this process never to adjust the paper manually. Once it's aligned it will always work for you. Also if we don't exit from the "menu" (light) with the "SHIFT" and "SEL" the printer will not save these settings (we always want to exit with these keys).

If the Order/Invoice doesn't align horizontally properly through the entire document
• Move the two paper feed tractors sideways.

If the print job doesn’t fit on a single page:
Locate the LPI settings and change them
• Enter Menu mode like was done to print the settings
• Press the "LF/GROUP" button to change the group
• Press the "?/ITEM" button to change to the item that you wish to
change
• Press the "TEAR/SET" button to change the setting
• Exit the command mode of the printer in the same method used to enter

If the printer does not start the next page on new page:
Adjust the “Auto LF” (menu) feature.

If  the printer indents every lines:
Adjust the “Auto CR” (menu) feature.

If  the font size is incorrect:
We can try the CPI feature on the front panel. Larger the CPI smaller the text. Or program the CPI permanent by the menu features. Or if we want a lot lines or chars on a single page ask us for “compress 10CPI” feature in PDS.

Settings Example: